University of Bridgeport

CS/CpE 460 Introduction to Robotics

Fall 2011

Project 2: Surveyor SRV-1 Blackfin Lane Detection.

Prepared by:

Tariq Alshugran
Qassim Bani Hani
Tamer Youssef

# Table of Contents

Table of Contents	2

Introduction	3

Connecting SRV-1 to the computer using wireless WiFi	3

Using RoboRealm Software with SRV-1	6

The Lane Detection Process	8

Preparing the lanes	8

The Lane Detection Code	9

References	17

# Introduction

The objective of this project is to utilize the SRV-1 robot camera to detect a lane and then move the robot along the lane. The images will be received from a the robot and then processed on a computer using a third party software then issue the robot a command to move along the lane. To do this, first we need to connect the robot to a computer utilizing the matchport WiFi of the robot, and then the image will be processed using RoboRealm software which will also issue the movement command.

# Connecting SRV-1 to the computer using wireless WiFi

- Turn on the SRV-1 robot so that the Matchport is powered. Start your wireless network manager program, and connect to the SRV1 adhoc network.

- Ping the robot IP address, in our case the IP address is 169.254.0.10. If the ping was successful and the data was received without any loss then the connection is successful.

- Run the SRV1Console and make sure that you can receive images as shown in the screenshot below, if the you are getting errors in the console as “Unable to connect” or “Wrote ‘I’” then move to the next step, otherwise the wireless connection was successful and the robot is ready to be used.

- If you are getting an error in the console like “Unable to connect” or “Wrote ‘I’” but still you can ping the port successfully, then the matchport WiFi connection is successful but the baud rate is not set correctly so that the matchport is unable to communicate with the processor, in such a case do the following:

  - Connect the Robot to USB as described in assignment 2.

  - Establish a connection on COM3 using a terminal program such as Br@y++ between the computer and the robot.

  - Issue the command xxx followed by Enter.

  - Then change the following:

Expert (5) -

For CPU performance, enter FF

For clk?, enter 81

Change MTU Size from 1400 to 1024

Skip the rest of the options

Channel 1 Serial (1) -

For Baudrate, enter -1

For divisor, enter 2

For flow control, enter 2

Further down, for FlushMode, enter 80

For Pack Cntrl, enter C0

For InterCh Time, enter 3

Skip the rest of the options

Save and exit (9)

  - Now disconnect the USB connection and try to follow step 1 – 3 again, the issue should be resolved and the wireless connection should be ready again.

# Using RoboRealm Software with SRV-1

In RoboRealm software there are two modules that can connect to SRV-1: SRV1 and SRV1b. The SRV-1 is to connect using COM port, while the SRV1b is used to connect using the WiFi. To start working with the SRV and RoboRealm follow these steps:

- Ensure that you computer is connected to the SRV-1 using wireless network as described previously. The SRV1b by default is configured for 192.168.0.15 in our case we need to change the IP address in the RoboRealm module IP address. Select the "Remember as Default" checkbox if you would like the current IP setting to be remembered by RoboRealm such that whenever the SRV1b is loaded the IP address will be auto-populated to the current setting. This ability allows you to not have to constantly change the IP address when loading in successive RoboRealm SRV configurations.

- Download and install the RoboRealm software from http://www.roborealm.com/registration/index.php and then run  RoboRealm.exe

- Insert in the SRV1b module. The quickest way to do this is to click on the Search tab in the upper left corner and type in surveyor. The module will appear in the list below your typing. Double click on that module to insert it into the pipeline. Note that there are TWO Surveyor_SRV modules. One without the 'b' which is the old system and one with a 'b'. Chose the one with a 'b'.

- After selecting the SRV1b module, click insert to add it to the modules panel.

- Once the module is inserted, edit the module properties to match the robot settings, in our case we have one camera so we disabled the stereo vision and inserted the correct IP address and the port number as show in the image below:

# The Lane Detection Process

The Surveyor SRV-1b is a great platform for experimenting in machine vision. With the wireless communication and built in camera the SRV-1b is a natural choice for machine vision processing using a PC.

## Preparing the lanes

In this case we are looking for yellow squares that define a lane for the robot to follow. The lane of squares is created using yellow pieces of thick paper cut into small pieces and placed approximately 6 inches apart. Care needs to be taken to place each square far enough from the previous square otherwise the robot will not be able to see the next square as it loses track of the previous one. The problem is that the robot camera is horizontal with the floor so it can’t detect the ground fully. Once the robot determines that no additional squares are present it will turn around and proceed back over the course. All the yellow squares are then placed on a black board 6 inches apart to create a road as the one shown in the image below:

From the robot's point of view the setup looks quite different than from an overhead view. This is largely due to the proximity of the camera to the ground and the lack of adequate lighting to provide enough contrast in the image. The bad lighting of the setup was done on purpose in order to review image processing techniques that can help in these kinds of environments.

## The Lane Detection Code

After loading the SRV1b module, you should be able now to see the video feed from the robot to the software.

Now we have a problem that the camera is horizontal with the floor, so it will not be able to see the lane on the board, to unwrap the frontal view of robot add the perspective module and tilt the image to show only the frontal view with a degree of 0.0099999998.

Then convert the image to gray scale to reduce the reflecting surface distortion by adding the GraySclae module.

Then add flood filters to show the lane trail:

Then crop the image to show only the immediate area immediately in front of the robot by adding crop module and set the values to (0, 0) – (320, 100)

Then add a BLOB filter to filter the exact location of the trail and remove background.

Now add the center of gravity module to find the center of the blob.

Finally add the code to control the motor motion. The code is written in VBScript language and saved in the VBScript module. The point we want the robot to move towards is contained in the COG_X and COG_Y variables. These variables are created by the Center of Gravity module.

To control the motors of the SRV-1b to move towards that point we need to just use the X coordinate and steer left if the point is to the left of the screen or right if the point is towards the right of the screen. Basically we want to control the robot to keep the X COG in the middle of the screen. This will then cause the robot to move along the defined trail and also attempts to keep as much of the significant part of the image (i.e. the squares) in view.

We use the VBScript module to create two motor variables that are then fed to the Surveyor SRV-1b module.

' determine the center of the screen
midx = GetVariable("IMAGE_WIDTH") / 2
' the amount of turning force or motor
' value difference that is used to 
' turn the robot.
factor = 2.5
' robot speed
speed = 155
' even out the motor values as one side can 
' be more powerful than the other
left_bias = 5
right_bias = 0
' get the COG_X that the COG module
' calculated
cogx = GetVariable("COG_X")
' if we see something then change direction
' accordingly
if GetVariable("COG_BOX_SIZE") > 10 then
' how far off the center screen is the 
' robot and how forceful does that
' turn need to be
spread = CInt((midx - cogx) / factor)
' set the motor values to be used in
' the robot control module
SetVariable "left_motor", speed - spread
SetVariable "right_motor", speed + spread
SetVariable "turn_status", 0
else
' if we don't see anything worth following
' we need to think about starting to turn.
' We first continue straight for a little bit 
' which ensures that the robot passes
' over the current square, we then turn
' to the right for a short bit to see if we
' missed a square outside the field of
' view and then we start doing the full
' turn. The variable turn_status keeps
' track of where we are during this
' sequence.
if GetVariable("turn_status") = "0" then
' have a little faith and keep going straight for a short time
  SetVariable "right_motor", speed
  SetVariable "left_motor", speed
  ' in 500 ms the turn_status will jump to the
  ' next step
  SetTimedVariable "turn_status", "2", 500
  SetVariable "turn_status", 1
elseif GetVariable("turn_status") = "2" then
' no blobs - turn one way for a little bit and then
' turn the other
  SetVariable "right_motor", speed+15 + right_bias
  SetVariable "left_motor", 255 - speed - 15 - left_bias
  ' in 1000 ms the turn_status will jump to the
  ' next step
  SetTimedVariable "turn_status", "4", 1000
  SetVariable "turn_status", 3
elseif GetVariable("turn_status") = "4" then
  SetVariable "left_motor", speed + 15 + left_bias
  SetVariable "right_motor", 255 - speed - 15 - right_bias
end if
end if

All the modules and the code are save to a robo file, please use the file laneDetection.robo to run the code and use the SRV-1 for lane detection.

# References

- http://www.surveyor.com/cgi-bin/yabb2/YaBB.pl?num=1257565328

- http://www.surveyor.com/cgi-bin/yabb2/YaBB.pl?num=1200515001

- http://www.surveyor.com/blackfin/SRV_setup_bf.html

- http://www.robotreviews.com/reviews/surveyor-srv-1-blackfin-robot-review

- http://www.pololu.com/docs/0J7

- http://www.surveyor.com/srvdownload/termv19b.zip

- http://www.surveyor.com/cgi-bin/yabb2/YaBB.pl?num=1196352263

- http://www.voxatec.com/downloads/RoboBrain_Manual.pdf

- http://www.roborealm.com/help/Surveyor_SRV1b.php